- Defining coastal hazard zones for setback

- lines A guide to good practice

- Doug Ramsay, NIWA Bronwen Gibberd, 4D Environmental Ltd Jin Dahm, Eco Nomos Ltd Rob Bell, NIWA

- 1.1 Purpose of this guide

- This best-practice guide has been developed to assist Local Authorities make sustainably- based decisions for the avoidance or mitigation of coastal hazard risks within their coastal land-use planning and development decision-making. It has been developed to help achieve the purposes of the RMA (1991) and Objective 5 and Policies 24-27 of the New Zealand Coastal Policy Statement 2010. The guide has been written to sit alongside and support the Ministry for the Environment guidance manual Coastal hazards and climate change: A guidance manual for local government (Ministry for the Environment, 2008). It also dovetails with the Pathways to Change guidance (Britton et al., 2012) in relation to developing and implementing strategic plans for coastal adaptation to climate change.

- It aims to provide concise guidance to enable:

- ⧠ A consistent and transparent framework for defining present and future coastal hazard exposure as an input to development setbacks to manage risk from coastal- related hazards and the effects of climate change.

- ⧠ Increased robustness and accounting of uncertainties within coastal hazard assessments.

- ⧠	Improved baseline information for incorporation of risk-based considerations within coastal land-use planning and development decision-making.

- ⧠	Achievement of the purposes of the RMA (1991) and Objective 5 and Policies 24-27 of the New Zealand Coastal Policy Statement 2010.

- ⧠	Appropriately defined requirements for consultants or resource-consent applicants conducting coastal hazard and risk analysis associated with coastal development.

- Following this introductory section, the guide is structured as follows:

- ⧠ Section 2 provides an overview of coastal hazard and setback zones in the New Zealand context, along with a summary of the framework for carrying out coastal hazard assessments and a checklist of good practice.

- ⧠ Section 3 provides guidance around approaches used in identifying and defining coastal erosion hazard zones.

- ⧠ Section 4 provides guidance around approaches used in identifying and defining coastal inundation hazard zones. This focuses primarily on storm-related inundation. For approaches to define tsunami hazard areas and integration into land use planning, see (Saunders et al., 2011).

- ⧠	Section 5: provides links to various technical resources and supporting information.

- Appendix 1 summarises the relevant provisions in the New Zealand Coastal Policy Statement, with Appendix 2 containing background information on Average Recurrence Intervals and Annual Exceedence Probabilities.

- P6

- 2.1 Coastal hazards and coastal hazardrisk

- 2.1.1 Introduction

- Taking a precautionary approach to planning new development, infrastructure and services to avoid coastal hazards over their intended lifetime is the most effective and sustainable long-term approach (MfE, 2008). This approach is relevant to all coastal development situations, from completely undeveloped coastal margins to high-density urban areas.

- A common approach where existing or proposed development is exposed to coastal hazards, is to apply information on potential coastal erosion in the form of coastal setbacks through a range of rules in planning documents, and, for inundation, specifying minimum floor and ground levels in land-development or engineering quality standards. In heavily developed areas, coastal hazard maps may be used simply for public education and awareness purposes, or to inform evacuation for storm-tide or tsunami warnings.  P7

- 2.2 The importance of the journey

- The process of defining coastal hazard zones and associated development setbacks is often complex, expensive, time consuming and iterative. There are a number of phases to any setback project, including scoping, design, calculation, review, mapping, community consultation, feedback, re-evaluation and implementation. Many coastal hazard setback projects receive the greatest resistance and are most prone to failure at the final stages of implementation. However, the resulting arguments typically centre on the early stages of the project, particularly the design. As outlined in the previous section, a common theme that came through in the consultations was that communication and cooperation between technical and planning staff was a major problem affecting effective setback design and application.

- More effort at the outset to improved planning through careful forethought and clear communication in both the scoping stages, and throughout the project can greatly improve the likelihood of an efficient and effective process, Table 1. P9

- 2.3 The process of defining development setbacks to

- incorporate coastal hazards

- Defining coastal hazard zones that are applicable for at least 100 years is not an exact science. Nor can the methodologies used, and expertise available, predict exactly what will happen in the future. Even with very good information there is always a level of uncertainty in the final coastal hazard prediction. The magnitude of this uncertainty increases with the planning timeframe, and even more so given the influence of future climate change and sea- level rise.

- The treatment of uncertainty in defining coastal hazard zones, and resulting setbacks is often a major source of difference between the results of different practitioners at the same location. The framework outlined in Figure 1 below, and discussed more fully in the following sections, aims to provide a process for a more consistent and transparent treatment of uncertainty and how it is communicated and incorporated within risk-based coastal land-use planning and development decision-making. P11

- The basic rationale applied within this guide is that:

- 1.	Coastal hazard zones should more effectively encapsulate and present the inherent uncertainties,

- to better inform development setback lines based on acceptable and appropriate levels of risk.  P12

3.1.2	Broad-scale coastal inundation hazard identification

- A broad scale assessment of coastal inundation potential can provide useful guidance for coastal management where such information is lacking. Such assessments do not provide sufficient detail for land-use planning decisions but they can provide a “first stage” or “red flag” to guide further more detailed hazard assessments at priority locations.

- A broad indication of areas where inundation may be an issue at a national scale, based on low-lying coastal margins are shown in the Figure 3 (left). Figure 3 (right) shows sections of the open coastline that are sensitive (relatively) to coastal inundation based on the national geomorphological classification.

- At a national level relative tsunami hazard was summarised in the Review of Tsunami Hazard and Risk in New Zealand (Berryman, 2005). Average recurrence interval tsunami wave heights for 100 and 500 years are shown in Figure 4.  P15

- 3.1.3	Building a qualitative understanding of coastal inundation behavior

- Assessment of coastal inundation hazard due to coastal storm events (i.e., not including tsunami and river floods) needs to give consideration of (MfE, 2008):

- ⧠	The interactions between the various coastal hazard drivers that can combine to produce inundation, the effects of climate change on these drivers, and how they influence inundation.

- ⧠	The dynamic nature of inundation over land, particularly the mechanism of how seawater inundates a certain area (flood pathways) and the storage potential of a flood area relative to the volume of inundating water flowing in to the area, and drainage capacity of the area.

- ⧠	The uncertainties associated with the assessment methods, data and information used.

- The development of conceptual behavioral models for a section of coast (often referred to as coastal behavioral units, compartments or cells) helps frame the understanding of the natural functioning of the coastal system as well as the pattern of human use and development and its influences on that system. Conceptual models are concise and ideally visually-stimulating illustrations that use symbols or drawings to depict the important features, processes and management challenges within a section of coast that “behaves” in a generally similar way (OzCoast, 2012).

- The Source-Pathway-Receptor-Consequence (SPRC) framework is one convenient way to consider the key drivers (and driver interactions), processes and controls on coastal inundation (both tsunami and storm-related) and how they impact on the range of the human and built environments within particular coastal margins (MfE, 2008). Example sources, pathways and receptors for coastal and tsunami inundation are shown in Table 2.  p17

- Commentary on the sea-level guidance:

- The risk assessment should be based on a broad consideration of the potential consequences

- (direct impacts, loss of assets and amenity) from different sea-level rise magnitudes on a specific

- decision, objective or issue. The particular sea-level rise adopted in each case should be based on

- the acceptability of the potential consequences and likelihood of that sea-level rise (=risk) and the

- potential future adaptation costs that may be incurred, especially if sea-level rise is higher than

- anticipated.

- Each risk assessment should also take into account the physical shore-type context (e.g., gravel,

- sandy or cliffed coasts) and the adjacent land-uses. In particular, upgrading existing development

- should be treated differently from new developments (‘greenfields’), where risk avoidance and a

- precautionary approach are paramount (e.g., Policies 3(2) and 25(b) of the New Zealand Coastal

- Policy Statement (NZCPS), DoC 2010; see Appendix 1) along with the need to recognise the

- permanency of such developments and that sea levels will continue to rise for possibly several

- centuries. Therefore in undertaking a risk assessment and appraising future adaptation for

- greenfield developments, sea-level rises well over 0.8 m should be considered. The MfE guidance

- (MfE, 2008a), as it stands, recommends assessing a range of sea levels, starting any appraisal with a

- 0.5 m rise (by 2090s) and with the “at least 0.8 m” as a minimum higher sea-level rise to consider

- when assessing future consequences. Using this set of two benchmark values is therefore a

- minimum to consider, but assessments should not to be limited to those values.

- Hence the risk assessment process, as recommended in the MfE guidance manual (MfE, 2008a), is

- an enduring approach, although it will need updating periodically in terms of planning timeframes.

- For example, the 2010 NZCPS requires assessments of hazards for “at least 100 years” (Policies

- 10(2)(a),24, 25). So already the range of sea-level rises that should be considered needs to take into

- account the presently recommended extension of 10 mm per year beyond 2100 e.g., the equivalent

- benchmarks by 2115 (nominally the next 100 years relative to the 1980–1999 average) would be for

- an assessment starting at a base value of 0.7 m (equivalent to 0.5 m rise by 2090s) and considering

- a range of possible higher values including at least a 1.0 m rise (was a 0.8 m rise by 2090s). Both

- these 2115 values have been rounded to the nearest 10 centimetres, taking into account the

- present guidance is for the 2090s decade with mid-point at 2095  P26

- Commentary on the sea-level guidance:

- The risk assessment should be based on a broad consideration of the potential consequences

- (direct impacts, loss of assets and amenity) from different sea-level rise magnitudes on a specific

- decision, objective or issue. The particular sea-level rise adopted in each case should be based on

- the acceptability of the potential consequences and likelihood of that sea-level rise (=risk) and the

- potential future adaptation costs that may be incurred, especially if sea-level rise is higher than

- anticipated.

- Each risk assessment should also take into account the physical shore-type context (e.g., gravel,

- sandy or cliffed coasts) and the adjacent land-uses. In particular, upgrading existing development

- should be treated differently from new developments (‘greenfields’), where risk avoidance and a

- precautionary approach are paramount (e.g., Policies 3(2) and 25(b) of the New Zealand Coastal

- Policy Statement (NZCPS), DoC 2010; see Appendix 1) along with the need to recognise the

- permanency of such developments and that sea levels will continue to rise for possibly several

- centuries. Therefore in undertaking a risk assessment and appraising future adaptation for

- greenfield developments, sea-level rises well over 0.8 m should be considered.

- The MfE guidance

- (MfE, 2008a), as it stands, recommends assessing a range of sea levels, starting any appraisal with a

- 0.5 m rise (by 2090s) and with the “at least 0.8 m” as a minimum higher sea-level rise to consider

- when assessing future consequences. Using this set of two benchmark values is therefore a

- minimum to consider, but assessments should not to be limited to those values.

- Hence the risk assessment process, as recommended in the MfE guidance manual (MfE, 2008a), is

- an enduring approach, although it will need updating periodically in terms of planning timeframes.

- For example, the 2010 NZCPS requires assessments of hazards for “at least 100 years” (Policies

- 10(2)(a),24, 25). So already the range of sea-level rises that should be considered needs to take into

- account the presently recommended extension of 10 mm per year beyond 2100 e.g., the equivalent

- benchmarks by 2115 (nominally the next 100 years relative to the 1980–1999 average) would be for

- an assessment starting at a base value of 0.7 m (equivalent to 0.5 m rise by 2090s) and considering

- a range of possible higher values including at least a 1.0 m rise (was a 0.8 m rise by 2090s). Both

- these 2115 values have been rounded to the nearest 10 centimetres, taking into account the

- present guidance is for the 2090s decade with mid-point at 2095  P37

- DEFINING COASTAL EROSION ZONESDefining coastal erosion zones

- 42

- 4.1 Introduction

- 4.1.1 Purpose

- In the context of this guide, the fundamental purpose of a coastal erosion hazard study is to quantify the extent of potential shoreline or cliff change over a specified future timeframe as an input in to coastal margin development planning.

- Typical data

- •	Generally based on readily available historical and environmental

- •	Field observations are useful to complement other forms of information but

- requirements:

- information. A wide range of qualitative information can be included such as

- historic photographs and maps, previous reports, information from past

- storm events (e.g. newspaper records), and anecdotal information from

- long-term residents.

- should be interpreted carefully given it is a single, or limited, point in time to

- avoid serious misinterpretation of longer-term coastal change.

- Limitations:

- •	Typically requires more detailed assessments to support land-use planning

- •	If it doesn’t include predictions of future coastal change then limited to

- decision-making.

- present-day hazard risk.

- Application:

- •	Identification of indicative locations or areas of coastline susceptible or

- •	Informing and raising public awareness of coastal hazard issues.

- 4.1.3	Building a qualitative understanding of coastal system behavior

- Quantifying how beach and cliff coastlines change, and how the patterns and rate of change may be influenced by climate change is extremely difficulty. Coastal change is a complex process where coastal and river hydrodynamics, morphology, geology, sediment supply/deposition and in some cases human modifications all interact over multiple timescales (MfE, 2008), Figure 21.

- Further complicating matters, these processes are subjected to both positive and negative feedbacks within the coastal system, again all of which operate on a number of different spatial and temporal scales, and are compounded at river, estuary or harbour entrances and associated spits. Assessment of future movement in estuary or harbour shorelines due to both climate and environmental human change are challenging.

- There is no one methodology or model that accommodates all the spatial and temporal considerations required to estimate how a coastline may change in the future over the decadal to centennial timescales of relevance to land-use planning.	Nor is the science of coastal processes and prediction at a level of understanding yet to incorporate all these interactions in a fully quantitative manner. Rather a range of tools, models and methods need to be applied within the context of expert judgment and drawing on similar analogues elsewhere (heuristic approach) to estimate long-term coastal change (Whitehouse et al., 2009). P45

- Q1You state(quote above. Has Shand done that?

- As with coastal inundation, the development of conceptual behavioral models for a section of coast (often referred to as coastal or cliff behavioral units, compartments or cells) helps frame the understanding of the natural functioning of the coastal system as well as the pattern of human use and development and its influences on that system.

- A conceptual model should aim to capture the key qualities of coastal-change behavior, giving qualitative insight into past shoreline changes and whether current shoreline behaviour and past changes are permanent, on-going or likely to reverse. In a coastal change context, it needs to capture what drives and controls both coastal planshape and cross-section profile response, including key sediment sources, pathways and losses, within the beach or cliff behavioural unit over differing timeframes.

- Fundamentally a conceptual model forms the basis for the choice of methodology adopted and provides a critical interpretation of the outputs of the analysis (Whitehouse et al., 2009). A well-defined coastal behavioural model also provides a tool for assessing the implications of different management approaches.  P462

- Q2 You state(quote bold above). Has Shand done that?

  - Quantifying shoreline change

- 4.2.1 Introduction

- Methods for assessing coastal change and prediction of future change in coastal morphology broadly fall in to three types (Whitehouse et al., 2009), Table 3.

- In practice, all approaches typically straddle the boundaries between the general model types. No process-based models include all the physics involved in simulating morphological change with components relying on behavioural representations, and in many cases the potential for change of state will need to be considered in simulating long-term, large scale coastal behavior.

- Furthermore, no one methodology or model accommodates all the spatial and temporal considerations typically required within a coastal hazard assessment (Figure 4). Hybrid approaches are a means of integrating methods or models that address different spatial or temporal scales.

- The choice of a particular approach for a particular situation will be influenced by the physical characteristics of the environment, the severity of the current or potential hazard risk, the volume and form of data available, computer modelling resources, and (sometimes most significantly) available budget.  P48

- In many open coast (particularly sandy) beach and estuary/tidal entrances, short-term erosion and medium term dynamic shoreline fluctuations can represent a significant portion of the overall coastal erosion hazard, for example:

- ⧠	Major storm events or sequences of storms where significant sand or gravel beach and/or dune erosion can be experienced as sand or gravel is redistributed offshore and/or alongshore. In quieter periods (weeks, months, years) between storms, sediment can return onshore rebuilding beach and dune levels.

- ⧠	Broader climatic patterns and cycles (such as annual summer/winter storm patterns, El Niño Southern Oscillation and Inter-decadal Pacific Oscillation) can drive annual inter-annual and inter- decadal patterns of erosion and recovery or accretion. These changes can occur over periods of decades but do not necessarily represent a permanent change in average shoreline position (Bryan et al., 2008).

- In some coastal settings, dynamic shoreline change can occur over decades and even centuries, often related to episodic events, such as eathquakes or volcanoes, resulting in a “pulse” of sediment supply to the coast (e.g. West Coast North Island) or subsidence of the coastline to change the relative sea level, for example the coast south of Napier. These cycles can be difficult to identify and predict, but have important implications for managing local subdivision and development. P51

- Assessing long-term shoreline change trends

- For the purpose of erosion hazard assessment, a long-term trend is defined as a progressive or net trend for shoreline change over the relevant planning horizon (typically 100+ years as required by the NZCPS, 2010).

- For cliffed and some estuarine coasts, shoreline change tends to be a one-way erosion process. However, for many soft open coast shorelines and close to tidal inlets, there can be both landward and seaward movements in shoreline positions, over varying spatial and temporal timescales, and it can be difficult to reliably separate long-term trends from dynamic variability as:

- ⧠	Dynamic variability is often large relevant to any long trends (Bryan et al., 2008).

- ⧠	Many/most shoreline position records or even aerial photographs are of insufficient time period to accurately extract dynamic shoreline change from long term trends

- with any certainty.

- Q3 You state(quote bold above). Has Shand done that?

- ⧠ Shoreline change data is often inadequate, for example temporally sparse or

- irregular data, or different or ambiguous shoreline features captured in surveys (for

- example high tide line, dune toe, or vegetation line).

- ⧠	Patterns of long-term change can be highly spatially variable.

- If the available record is too short, the mis-interpretation of dynamic fluctuation as long- term shoreline change can result in significant error, particularly if rates of historic change are then used to project potential future change. Where dynamic fluctuations in shoreline position occur, identifying and separating long-term change from such fluctuations is critical if such information is to be used to define future coastal hazard zones.

- Analysis of shoreline features extracted from historic aerial photographs, surveys and maps is the method most widely used in New Zealand to quantify long-term shoreline change and medium term fluctuations (Table 5). Note, old surveys typically captured mean high water and if used it is necessary to go back to the original survey books to verify how mean high water was defined. Analysis of beach profile datasets can also contribute where a sufficient length of dataset is available, but there are few such datasets with greater than 20 to 30 years of records.  P53

- Irrespective of methodology used, good practice process in determining a long-term trend includes that:

- ⧠	The trend is considered in the context of any short and medium-term dynamics. The data used needs to adequately represent the different states of the shoreline, and is not heavily weighted toward a particular shoreline state (due to the temporal coverage of the data), and therefore suggest and apparent trend that may not be accurate. Ensuring that results from analysis of datasets concur with the conceptual understanding of the coastal system.

- ⧠	The length of the dataset used to calculate long-term trends should be in excess of 50 years and ideally be at least equivalent to the length of the intended planning period as well as being longer than any known cycles/patterns of shoreline change.

- ⧠	Aerial and satellite imagery, survey and map information are properly geo-rectified and where necessary ortho-rectified. The process undertaken to properly position historic datasets against a common base map or image, and to correct image distortions, should be clearly defined.

- ⧠	The baseline from which all shoreline feature measurements are made is clearly defined.

- ⧠	Consistent natural features are captured from each of the historic datasets (such as the cliff edge, vegetation line or dune toe).  P58

- 4.3 Defining future coastal erosion hazard

- 4.3.1	Current approaches

- Many different approaches have been adopted by New Zealand practitioners to predict potential future coastal erosion hazard. Despite a variety of terminology used, all are typically a variation of the same general approach involving a simple addition of some or all of the following components:

- ⧠ Extrapolation of the calculated long-term erosion rate over the future planning timeframe.

- ⧠	Maximum assumed potential storm erosion and/or other fluctuations.

- ⧠	A dune, cliff or other setback factor.

- ⧠	Additional retreat of the shoreline due to sea-level rise or other climate change

- influence.

- ⧠ A factor of safety which is typically used to incorporate uncertainties in the

- methodology and is usually defined based on heuristic reasoning.

- Dune, cliff or other setback factors

- In addition to the long term erosion, storm erosion and other short to medium term fluctuations typically defined from analysis of measured data or numerical modelling, most practitioners also apply some form of “dune” or stable slope factor:

- ⧠ For dune coasts this provides sufficient additional width to provide for the unconsolidated nature of the sediments and the tendency for a facetted dune face to erode to a stable angle following storm erosion. This factor has traditionally been calculated by observing the natural angle of repose of loose sand or applying a universal value (typically approximately 30 degrees).

- ⧠ For gravel or shingle beaches this is often related to width to the beach crest position or the backshore of the beach crest.

- ⧠	For cliffs this is often related to the cliff height and is based on the geomorphological nature of the cliffs and angle of stable slope after episodic failure events.

- Incorporating the impacts of sea-level rise

- The response of a shoreline to climate change will primarily depend on sea-level rise, the wave climate, and for beaches, the sediment budget (the ongoing supply, redistribution and losses of sediments).

- Application of a well-constructed conceptual model of the coastal or cliff behavioural unit can also help frame an understanding of the potential influence that sea-level rise and/or other climate factors will have on the future cross-sectional profile and planshape response of a section of coast (Box 13).

- Q4 You state(quote bold above). Has Shand done that?

- Similar types of coast will not always respond in the same way and consideration needs to be given to not just change in the hazard drivers (which can

- The most common approach is typically based on the “Bruun Rule” which is applicable to open-coast sandy beaches, Figure 31, (Bruun, 1962). p68

- The many limitations and assumptions associated with the Bruun Rule include:

- ⧠	No consideration of longshore sediment transport or gain or loss of sediment to the beach system.

- ⧠	The shore profile is considered entirely beach, and there are no lithological controls

- ⧠	The wave climate is steady and consequently the (average equilibrium) beach profile

- does not change.

- ⧠	The magnitude of retreat suggested by the Bruun Rule is very sensitive to the slope

- of the beach and the estimated closure depth. P68

- In selecting, using and applying an approach to incorporate sea-level rise considerations for shoreline change, the following must be clearly considered and communicated:

- ⧠	A range of sea-level scenarios need to be assessed and the sensitivity of the model predictions ascertained.

- ⧠	The methodologies selected need to be informed by a conceptual understanding of how the beach system may change with sea-level rise and climate change.

- ⧠	Where appropriate a range of methodologies should be investigated and applied the variability in response considered.

- ⧠	What assumptions are being made, limitations, range of validity and uncertainties there are for the methodologies used and associated impacts on the intended use of the approaches.

- In many cases, the methodologies applied often indicate that erosion predicted to occur in response to sea-level rise will be the largest single component influencing the magnitude of future coastal erosion.  P69

- Quantifying uncertainty to underpin risk-based decision-making

- It is now well recognised that quantification of this uncertainty can bring much richer level of information to underpin decision-making, and is crucial where a risk-based approach to hazard decision-making is required, as it is in New Zealand.  P704

- Q3 You state(quote bold above). Has Shand done that?

- The approach outlined below aims to build on the current deterministic approach commonly used in New Zealand, and the expertise and experience held by practitioners, to determine future coastal change, but to apply it within a pragmatic stochastic (probabilistic) way, as described by Cowell et al., (2006). The general approach is outlined below:

- ⧠	Uncertainty is captured in each parameter or variable where it exists as individual probability distributions.

- ⧠	These probability distributions can be derived either directly from analysis of data where it exists, from simple heuristic reasoning based on experience, or a combination of the two. At the most basic level a simple triangular distribution can be defined that contains a best estimate and lower and upper bounds of the parameter (i.e. it assumes that the actual value lies somewhere within the bounding range).

- ⧠ The peak of each distribution (i.e. the most probable value of a parameter or variable) corresponds to the calibrated, or “best” estimate of the parameter or variable in the deterministic approach.

- ⧠ Simulation techniques, such as Monte Carlo methods, are then used to repeat sample each of the probability distributions of each input variable to generate many thousands of possible outcomes from which probabilities of the different outcomes (coastal erosion hazard) are derived. P70

- The approach outlined below aims to build on the current deterministic approach commonly used in New Zealand, and the expertise and experience held by practitioners, to determine future coastal change, but to apply it within a pragmatic stochastic (probabilistic) way, as described by Cowell et al., (2006).

- The general approach is outlined below:

- ⧠	Uncertainty is captured in each parameter or variable where it exists as individual probability distributions.

- ⧠	These probability distributions can be derived either directly from analysis of data where it exists, from simple heuristic reasoning based on experience, or a combination of the two. At the most basic level a simple triangular distribution can be defined that contains a best estimate and lower and upper bounds of the parameter (i.e. it assumes that the actual value lies somewhere within the bounding range).

- ⧠ The peak of each distribution (i.e. the most probable value of a parameter or variable) corresponds to the calibrated, or “best” estimate of the parameter or variable in the deterministic approach.

- ⧠ Simulation techniques, such as Monte Carlo methods, are then used to repeat sample each of the probability distributions of each input variable to generate many thousands of possible outcomes from which probabilities of the different outcomes (coastal erosion hazard) are derived. P74

- 4.4.3	Zones to inform lines

- How coastal erosion hazard information is presented is vitally important for effective development setback decision-making. It is one where there needs to be a shift in current practices to enable a more transparent process and to reduce the political and community conflicts that commonly occur.

- Current approaches to presenting coastal erosion hazard information tends to present both the coastal erosion hazard information and the derived setback in the form of one feature, typically a line, or a series of lines on a map, for example representing an area of immediate coastal erosion hazard and the erosion hazard over the next 50 and 100 years (Healy, 2005). Whilst this provides a precise outcome (a defined line on a map) that is typically desired by decision-makers, it presents a precision in the context of the erosion hazard that is not justifiable as it does not communicate the associated uncertainties that are inherent in these future predictions. As discussed above, this frequently leads to the misunderstanding where a coastal hazard zone is considered to “constitute a “magical” safety zone immediately on one side of it, and a zone of “total hazard” or impending destruction on the other” (Healy, 1993).

- Rather, as discussed in the previous section, the presentation of coastal erosion hazard information must be able to encapsulate that:

- ⧠	Coastal erosion hazard reduces with distance from the coastline.

- ⧠	Predictions of future coastal erosion hazard incorporate considerable uncertainty.

- ⧠	Uncertainty of future coastal erosion hazard predictions increases with increasing

- timeframe of the prediction.

- ⧠	This uncertainty needs to be clearly presented to, and understood by all, that make

- decisions from coastal erosion hazard information.

- Both zones and lines are required. Presenting coastal erosion hazard information and defined development setbacks are two distinct steps that need to be presented individually where:

- ⧠	Coastal erosion hazard is presented as a zone that fully encapsulates the range of probabilities of erosion occurring over a specified future timeframe.

- ⧠	Development setbacks are presented as a line that is derived from: •A consideration and acceptability of likelihood or erosion occurring within a

- specific timeframe,

- The resulting potential risks (and acceptability of these risks based on

- community, planning and management aspirations) to existing or potential assets and values located within this coastal hazard zone. P78